# Comprehensive Workflow Documentation: Autonomous IFF Mode S Signal Degarbling

Target Hardware: ZedBoard (Zynq-7020) + Analog Devices AD9361 Transceiver Toolchain: MATLAB Deep Learning HDL Toolbox, Xilinx Vivado (2024.2)

## Overview

This document outlines the hardware-software co-design pipeline for deploying a custom deep learning model to process and degarble Identification Friend or Foe (IFF) Mode S signals. The system is designed to run autonomously on the Zynq-7020 SoC, utilizing the Programmable Logic (PL) for high-speed neural network inference and the Processing System (PS) for memory management and control.

## Phase 1: Model Definition & Synthetic Data Generation

To efficiently process the 1090 MHz Mode S preambles (sampled at 16 MSPS by the AD9361), a lightweight 1D Convolutional Neural Network (CNN) was designed.

Data Structure:

- Input Size: [128 1 2] (128 time samples, 1 spatial dimension, 2 channels for In-Phase and Quadrature baseband data).

- Classes: Clean Preamble, Garbled (Overlapping), Noise.

Synthetic Data Generation: Because physical RF data collection is subject to real-world thermal noise and multipath fading, a highly representative synthetic dataset was generated using MATLAB.

- Clean Pulses: Simulated four 0.5 µs pulses with random carrier phase and AWGN (10-30dB SNR).

- Garbled Pulses: Injected secondary overlapping preambles (FRUIT) with random time delays and amplitudes.

- Noise: Pure Additive White Gaussian Noise.

- Dataset Split: The data was explicitly partitioned into 70% Training, 15% Validation, and 15% Testing, and saved to mode_s_synthetic_data.mat.

## Phase 2: Training & INT8 Quantization

The 32-bit floating-point CNN was trained using the Adam optimizer. However, a floating-point network demands excessive DSP slices and Look-Up Tables (LUTs) that exceed the capacity of the ZedBoard's Zynq-7020 chip.

INT8 Quantization Workflow: To fit the model into the Programmable Logic, it was compressed to an 8-bit integer format using dlquantizer.

- Calibration: The 15% Validation set was fed through the network to calculate dynamic scaling factors, preventing signal clipping.

- Hardware Targeting: The quantizer environment was strictly set to 'FPGA' to ensure compatibility with Xilinx DSP48E1 slices.

- Result: The quantized model (quantObj) maintained near-identical accuracy to the floating-point model while shrinking the memory footprint to roughly 316 kB, allowing weights to be efficiently loaded via DDR memory. The object was saved to mode_s_quantized_fpga.mat.

## Phase 3: Hardware constraints & Toolchain Setup

Generating the VHDL IP core required explicitly linking MATLAB to the Vivado compiler and handling extreme resource bottlenecks.

Crucial Configurations:

- Tool Path Linking: MATLAB required explicit knowledge of the Vivado installation directory via hdlsetuptoolpath.

- Version Bypassing: Vivado 2024.2 required an override flag (AllowUnsupportedToolVersion = true) via the hdlcoder.WorkflowConfig object.

- Resource Optimization: The default MATLAB deep learning processor allocates resources for massive image processing (e.g., 227x227 inputs). To fit the ZedBoard's 220 available DSP slices:

  - ConvThreadNumber and FCThreadNumber were reduced to 4.

  - BRAM buffers were shrunk to match the [128 1] signal window.

  - Custom Math Modules were disabled.

  - ProcessorDataType was forced to 'int8' and UseVendorLibrary to 'off'.

Objective: Resolve compilation errors encountered while generating and compiling a Deep Learning HDL (dlhdl) IP core for an INT8 quantized Convolutional Neural Network.

## 1. Initial Overview & Problem Statement

The deployment pipeline was failing during the hW.compile(); step. The original configuration attempted to pass mismatched or corrupted hardware constraint variables, preventing the MATLAB Deep Learning HDL Toolbox from generating the network software binaries for the ZedBoard.

## 2. Issue Log and Resolutions

### Issue 1: Target Platform Mismatch

- Problem: The initial workaround script (compile_fix.m) targeted the Xilinx Zynq ZC706 evaluation kit, which did not match the physical hardware being used (ZedBoard xc7z020).

- Resolution: Switched the TargetPlatform to 'Generic Deep Learning Processor' and explicitly forced the synthesis tool properties to match the ZedBoard (ChipFamily = 'Zynq', DeviceName = 'xc7z020', PackageName = 'clg484').

### Issue 2: Invalid Workflow Parameter ('ProcessorConfig')

- Problem: The script attempted to pass a raw dlhdl.ProcessorConfig object directly into dlhdl.Workflow. MATLAB threw an error because dlhdl.Workflow strictly expects a 'Bitstream' parameter pointing to a generated .mat file.

- Resolution: Restructured the workflow to first run dlhdl.buildProcessor to generate a fresh mode_s_ip.mat constraint file inside the dlhdl_prj directory, and then pass that file path to the workflow object.

### Issue 3: Stale Cache and Intermediate Files

- Problem: Leftover files from previous failed Vivado builds (OP0tbResult.mat, AXIOutData.mat, etc.) and the old dlhdl_prj directory risked contaminating the new build process.

- Resolution: Implemented a pre-build cleanup sequence to wipe the workspace and delete intermediate simulation files before running the compiler.

### Issue 4: Out of Memory Error (0x0 Bytes Available)

- Problem: The final compilation failed with the error: Network requires 0x4F000 bytes of memory space, which is larger than the available external memory of 0x0 bytes. Because we used a "Generic" processor target to bypass validation, MATLAB defaulted the board's external RAM to 0 bytes.

- Resolution: Utilized a dlhdl.Bitstream mapping object to explicitly define the ZedBoard's memory architecture. We set the MemoryAddressRange to '20000000' (512 MB of DDR3 RAM), overriding the 0-byte default and allowing the 316.0 kB network to compile successfully.

## Objective

To generate the base hardware block design for the ZedBoard and AD9361 (FMCOMMS3) using the official Analog Devices (ADI) HDL repository. This reference design provides the necessary DMA, SPI, and clocking infrastructure into which the custom IFF Deep Learning IP core will be integrated.

## 2. Challenge: Identifying the Build Process

Issue: The Vivado .xpr project file does not exist natively in the downloaded ADI GitHub repository. Solution: The ADI repository uses Tcl automation to dynamically generate the project, instantiate IP cores, and draw the block design.

- Execution: Instead of clicking "Create Project," the developer must open the Vivado Tcl Console, navigate to the specific board directory (hdl/projects/fmcomms2/zed), and execute: source ./system_project.tcl.

## 3. Challenge: Vivado Hard Crashing

Issue: When attempting to run the system_project.tcl script using Vivado 2024.2, the Vivado software violently crashed (instantly closing the application without generating an error log).

Investigation & Root Cause Analysis:

- Script Inspection: Reviewed the system_project.tcl script to locate version constraints.

- Environment File Check: Discovered that ADI migrated their version dependencies to a global adi_env.tcl file.

- Version Mismatch: The environment file explicitly declared set REQUIRED_VIVADO_VERSION "2025.1".

- The Crash Mechanism: The user was operating on the bleeding-edge main branch of the ADI repository (built for 2025.1). When Vivado 2024.2 attempted to parse and dynamically auto-upgrade the newer 2025.1 IP core XML structures in the background, the internal C++ IP Packager engine panicked, resulting in an unhandled exception and a hard crash.

## 4. Strategic Decision: Version Selection ("The Golden Path")

Because Analog Devices did not publish an official hdl_2024_rX release branch, the project faced a fork in the road:

- Option A (The "Follow the Code" Route): Keep the main branch code and upgrade to Vivado 2025.1.

- Option B (The "Stable Legacy" Route): Downgrade the HDL code to the hdl_2023_r2 branch and install Vivado 2023.2.

The Engineering Rationale for Option B: The main branch (requiring Vivado 2025.1) is an active, daily-development workbench for ADI engineers. Their current focus is on next-generation silicon (e.g., Xilinx Versal, RFSoC). Because the ZedBoard (Zynq-7000) is legacy hardware, core DMA or interconnect changes made for newer boards frequently break older board compilations on the main branch without ADI immediately noticing.

To avoid spending weeks debugging broken Tcl scripts or broken data paths, the project standardized on Option B. The hdl_2023_r2 branch is a "frozen," QA-tested, certified release where the ZedBoard and FMCOMMS3 are guaranteed to compile and route flawlessly.

## 5. Resolution

- The AMD/Xilinx Vivado 2023.2 toolchain was installed.

- The corresponding hdl_2023_r2 release branch was pulled from the Analog Devices GitHub.

- The Tcl script (source ./system_project.tcl) was executed natively in the matching Vivado version.

- Result: The automated script ran successfully, generating the complete physical hardware architecture, unlocking the next phase of the project: surgical integration of the custom AXI4-Stream neural network.

### Project Log: ZedBoard AD9361 Deep Learning Integration

Phase 1: Base System Generation and Environment Setup

- Objective: Compile the Analog Devices (ADI) IP libraries and generate the default FMCOMMS2 block design in Vivado.

- Execution: * Ran the ADI Tcl build scripts to compile necessary IP blocks (ad9361, dmac, cpack, upack, etc.).

  - Successfully executed system_project.tcl to stitch the base ZedBoard block design together.

  - Generated the initial verification bitstream to confirm the hardware baseline was 100% functional.

Phase 2: Custom Deep Learning IP Integration

- Objective: Import and wire up the custom MATLAB HDL Coder neural network (mode_s_ip_0).

- Execution:

  - Added the exported MATLAB HDL folder to the Vivado IP Repository.

  - Instantiated the mode_s_ip_0 core onto the system.bd canvas.

  - Utilized Vivado's Connection Automation to route the IP.

  - Architectural Note: The IP was configured with AXI4 Memory-Mapped interfaces rather than AXI4-Stream. This established the system as a "Memory Co-processor" (batch processing via DDR RAM) rather than an inline real-time streaming filter.

Phase 3: Resource Optimization (The "Diet")

- Objective: Resolve initial LUT over-utilization by removing unnecessary factory peripherals.

- Execution:

  - Initial synthesis revealed the design required 56,878 LUTs, exceeding the Zynq 7020's physical limit of 53,200.

  - Performed a targeted deletion of high-resource, non-RF IPs from the block design.

  - Removed video processing blocks: axi_hdmi_core, axi_hdmi_dma, axi_hdmi_clkgen.

  - Removed audio processing blocks: axi_spdif_tx, axi_i2s_adi.

Phase 4: Resolving Ghost Constraints and Port Mismatches

- Objective: Clean up residual code left behind by the deleted HDMI and Audio blocks to allow successful Place and Route.

- Execution:

  - Physical Constraints: Commented out physical pin assignments for hdmi, spdif, and i2s in the zed_system_constr.xdc file to prevent Place and Route (place_design) crashes.

  - Top-Level Wrapper: Edited the handwritten system_top.v file to comment out the deleted audio/video wires and instantiations.

  - Block Design Canvas: Manually deleted floating external port labels (e.g., hdmi_data, i2s_mclk) from the visual canvas to stop Vivado from automatically rewriting them into the system_wrapper.v auto-generated file.

  - Cache Clearing: Repeatedly used Reset Output Products and Reset Runs to force Vivado to drop corrupted, cached synthesis files (specifically an AXI Interconnect auto_pc ghost port error).

Phase 5: Final Area Optimization

- Objective: Squeeze the final design into the remaining silicon after a near-miss (53,668 LUTs required vs. 53,200 available).

- Execution:

  - Changed Synthesis Strategy to Flow_AreaOptimized_high.

  - Changed Implementation Strategy to Area_Explore.

  - Result: The aggressive Vivado optimizations caused the logic footprint to increase to 56,596 LUTs, confirming a hard physical silicon limit.

### Problems Faced and Solutions

| Problem Encountered | Cause | Solution Applied |
| --- | --- | --- |
| Missing util_clkdiv Error | The ADI Tcl script could not find the clock divider directory during the initial library build. | Corrected the directory path in the Tcl console to point to the hidden subfolder: library/xilinx/util_clkdiv. |
| Initial LUT Over-Utilization | The default FMCOMMS2 design + custom deep learning IP required more logic cells than the Zynq 7020 possesses. | Deleted resource-heavy, unused HDMI and Audio IP blocks from the block design. |
| place_design Error | The Vivado Placer was looking for physical HDMI/Audio pins that no longer existed on the block design. | Commented out all associated set_property PACKAGE_PIN lines in the zed_system_constr.xdc file. |
| s_axi_arregion Synthesis Crash | Vivado attempted to use a cached AXI Interconnect file from before the block design surgery. | Right-clicked the block design in the Sources tree and executed Reset Output Products to wipe the cache, then regenerated them. |
| system_top Port Mismatches | The handwritten system_top.v file was trying to map wires to block design ports that were deleted. | Opened system_top.v and commented out the specific wire instantiations (Lines 194-203, 222). |
| system_wrapper Auto-Gen Errors | Vivado kept auto-generating HDMI/Audio ports in the wrapper because the physical output pins were left floating on the canvas. | Opened system.bd, highlighted the floating blue port labels on the far right edge of the screen, and deleted them. |
| Final LUT Over-Utilization (Hard Limit) | The mode_s_ip_0 neural network is mathematically too large to fit on the 53,200 LUTs of the ZedBoard, regardless of Vivado routing optimizations. | Pending Solution: The model must be shrunk at the source using MATLAB's Deep Learning HDL Toolbox (implementing INT8 Quantization and increasing the Resource Sharing/Folding Factor). |

Final script to prepare environment

# 1. Define the base path set repo_dir "A:/hdl-2023_R2_p1" # 2. Exhaustive list of required libraries for fmcomms2_zed # (Video/Audio IPs are included solely to satisfy the ADI base script) set required_libs { "util_cdc" "util_axis_fifo" "axi_dmac" "axi_clkgen" "axi_hdmi_tx" "axi_spdif_tx" "axi_i2s_adi" "axi_sysid" "sysid_rom" "util_i2c_mixer" "util_wfifo" "util_rfifo" "util_pack/util_cpack2" "util_pack/util_upack2" "axi_ad9361" "xilinx/util_clkdiv" } # 3. Build each library foreach lib $required_libs { puts "=========================================" puts "Building $lib..." puts "=========================================" cd $repo_dir/library/$lib # Extract the base name of the IP (e.g., util_cpack2 from util_pack/util_cpack2) # Also handles the xilinx/util_clkdiv subfolder cleanly set ip_name [lindex [split $lib "/"] end] # Source the ADI IP packaging script source ./$ip_name\_ip.tcl } # 4. Build the actual Zedboard project puts "=========================================" puts "Libraries built. Building FMCOMMS2 ZedBoard Project..." puts "=========================================" cd $repo_dir/projects/fmcomms2/zed source ./system_project.tcl